I am an atmospheric scientist (with coding experience in Python) working on solar radiation measurements and trace gas retrievals and I am starting a new project.

Background:

The retrieval of trace gas amounts from spectral solar measurements is an important technique in remote sensing of the atmosphere for satellite validation and air quality monitoring.

The usual way is to characterize the instrument (a spectrometer system that measures the direct solar irradiance) in the lab (pixel to wavelength relation, non linearity etc) to be able to convert the raw spectral data (L0) to L1 (corrected counts or absolute units of irradiance) and then do some sort of DOAS fit to retrieve atmospheric parameters (e.g. NO2 and O3 total columns).

Goal:

The goal here is to design an artificial neural network architecture that explicitly models the physics of each step in the complete process from the light of the sun to the electronic signal in the detector. In more detail: From the extraterrestrial solar spectrum, the absorption and scattering processes in the atmosphere, to the instrumental processes that result in the recording of a raw spectral signal that is read from the diode array detector.

So the question is if we could use a novel scheme to learn the individual contributions (sun, atmosphere, spectrometer) from a large number of measurements.

Method:

(I have no experience in setting up ANNs, so I will describe what i think could be a way)

So the series of processes would be modelled individually but part of an architecture that would be trained or self learning as a whole.

So we start with the extraterrestrial spectrum (the sun), which is constant, then the atmosphere is modelled by the de-beer lambert law (I=I0*exp(sum(tau*amf))) where tau is the optical depth (tau = xsection * total column, for each gas), all terms are a function of wavelength (spectral). This results in a unconvoluted “L1” spectrum which enters the instrument. The instrument convolutes the spectrum and applies a nonlinear conversion function and yields a raw “L0” spectrum.

So the architecture would look like: constant spectral input, atmosphere blackbox, instrument blackbox, spectral output. We have a lot of example data for spectral data that we use for training. I guess the system can be set up in both directions, L0 input - solar extraterrestrial output or vice versa.

The key ideas (and constraints) are as follows:

The system would not be trained on retrieved total columns but only using the physical constraints and self consistency. (if that is even possible, but that would ideally be the goal)

The data sets of L0 spectra are time series, a spectrum every couple of minutes during daylight, for one or more years. Each day, the sun rises and sets, so the spectral absorption of trace gases is “modulated” by the airmass factor (amf = 1/cos(solar zenith angle)). And the total column amount of each trace gas should be uncorrelated with the amf.

Implementation:

The system will be implemented using standard python libraries, computational recourse are not an issue (using cloud resources)

Steps include:

- System architecture design

- Implementation in python

- data preparation

- Training

- Visualization of model components

- validation

System design:

Solar extraterrestrial spectrum (Lx)

Spectrum after atmosphere (L1)

Spectrum after instrument convolution (L1’)

Spectrum after instrument electronics (L0)

L1 = f_atmos(Lx)  -> de beer-lambert: L1 = Lx*exp(sum(tau*amf))

L1’ = f_conv(L1)

L0 = f_instr(L1’)

Tau = xs * tc

De beer-lambert: L1 = Lx*exp(sum_i(tau_i*amf))

Tau_i = xs_i * tc_i

Lx: extraterr solar spectrum

Xs:crosssections, tc: total columns, sum index i runs over n gases

Analogy:

Evaluate the intensity of individual instruments (e.g. violin, guitar, etc, each has a specific frequency distr.) from a very noisy audio recording of an orchestra. In addition the intensity of the whole orchestra is modulated periodically, the distracting noise stays the same.

Refined implementation scheme:

More precise definition of the system to be trained.

Input: L0 raw spectra (t_int)

Output: L1 spectra (on regular wavelength grid)

L1 = f_instr(L0)

Lossfunction : RMS error of measured L1 to modelled L1 = Lx*exp(sum(tau*amf))

Where tau = xs * tc and tc is a variable and should be “fitted”.

The transformation from L0 to L1, f_instr, describes a spectral detector with readout electronics. It should consider the following steps of transformation:

-Dark current (subtract a variable constant)

-Fixed structure in dark current (subtract a 2048 array with small structure)

-Non linearity (L1 = L1 + a*L1^2 + b*L1^3+...)

-Stray light correction (multiplication of SL matrix)

-Conversion from cts to cts/s (L1 / t_int)

-Absolute calibration from cts/s to w/m2/nm (L1 = L1 * abs_cal, 2048 array)

-Filterfunction (L1 = L1 * filterfunction(FWpos), each filter wheel position(FWpos is input, integer) has a specific filter function, like attenuation or bandpass)

-polynomial wavelength dispersion (could be part of the convolution?)

-Convolution (with wavelength dependent slit function)

Except for the dark current (which may change from spectrum to spectrum) all other corrections are (nearly) constant properties of the instrument which should be learnt.

L0 is an array of 2048 pixel, L1 is an array of length of the regular wavelength grid (e.g. np.arange(300,500,0.2))

More refined formulation of the scheme and architecture.

We have two systems we need to characterize : f_inst and f_atmos, for the instrument and the atmosphere. F_inst is the instrument transformation “black box” L1 = f_instr(L0) and f_atmos is the de-beer lambert law L1 = Lx * exp(-sum_i ( xsec_i * totcol_i * amf ) )

f_instr:

Input: L0 raw spectra (1D arrays of length 2048 for one day), t_int and filter position, both integers

Output: L1 spectra on regular wavelength grid, (a series 1D arrays of length 2048 for one day)

F_atmos:

Input: Lx (extraterrestrial spectrum) , xsections for a number of gases (1D arrays of length 2048), amf (float)

Output: modelled L1_m (a series of 1D arrays of length 2048 for one day), totcol_i for ith gas (float) including an uncertainty or confidence parameter.

Loss function, spectral part and daily part

Loss = Loss_spec + Loss_day

Loss_spec = f(L1,L1_m) : minimized differences

Loss_day = f(totcol_i,amf) : minimized correlation of totcols and amf over day

So the Loss function is defined over a day (each with ~100 L0 spectra), and the training data consists of multiple days (~300?).